GARMIN GTX 330

11.TRANSPONDER CODE SET BUTTONS

12.VFR BUTTON

1.ON BUTTON

2.ALT BUTTON

3.OFF BUTTON

4.TRANSPONDER CODE

5.ON/OFF ANNUNCIATOR

6.FUNC BUTTON

7.CRSR BUTTON

8.CLR BUTTON

9.START/STOP BUTTON

10.DIM/BRT BUTTONS

13.IDENT BUTTON

14.STBY BUTTON

The Garmin GTX 330 transponders primary function is to transmit a 4 digit code that is used to locate the aircraft. The code is set using the TRANSPONDER CODE SET BUTTONS. This is done by clicking the desired numbers directly, and does not involve the use of any other buttons. In addition, there is a VFR BUTTON that sets the code to 1200 when clicked.

The GTX 330 transponder also provides the user with altitude and timer information. These functions are shown on the right side of the screen (see figures below). Clicking on the FUNC BUTTON sequentially displays pressure altitude, flight time, altitude monitor, outside ambient temperature and density altitude, a count-up timer, and a count-down timer.

1.  ON BUTTON:  Powers the transponder unit on. Selects Mode A. In this mode the

unit will respond to interrogations, but will not receive altitude information.

2.  ALT BUTTON:   Powers the transponder unit on. Selects Mode A and C. Both

interrogation and altitude information will be received.

3.  OFF BUTTON:  Turns the transponder off.

4.  TRANSPONDER CODE:  Digital display of the set transponder code.

5.  ON/OFF ANNUNCIATOR:  When lit indicates the transponder is on.

6.  FUNC BUTTON:  The function button is used to scroll through the different function

screens on the right side of the screen (screens listed above).

7. CRSR BUTTON:  This button has 2 functions. It is used to cancel the

transponder code and count-down entries, and to allow input data in the count-

down mode. Clicking the CRSR BUTTON when in the count-down screen allows a

time value to be entered using the 9 number buttons at the bottom (instead of changing

the transponder code). The count-down timer must be stopped and reset (click on CLR

BUTTON) before entering a new value.

8.  CLR BUTTON:  Resets the flight and count-up timers to 0, and resets the count-

down timer to the entered value. Also used to cancel previous keypress entries during

transponder code and count-down value selections.

9.  START/STOP BUTTON:  Starts and stops the altitude monitor, count-up, count-

down and flight timers. Restarting the timers will start at the value when stopped.

10. DIM/BRIGHT BUTTONS:  Clicking on the #8 button will decrease the display

brightness and clicking on the #9 button will increase it. Default is set to the brightest

value. They are also used in entering the count-down value in the count-down screen.

11. TRANSPONDER CODE SET BUTTONS:  Number buttons 0-7 are used to

enter the transponder code. They are also used to enter the count-down value in the

count-down screen. As noted above, the CRSR BUTTON must be clicked to enter

timer value.

12. VFR BUTTON:  Sets the active transponder frequency to the VFR frequency 1200

directly.

13. IDENT BUTTON:  Activates a special identification pulse (SPI) for 18 seconds to

distinguish your aircraft from others on the air traffic controller's screen... The

word IDENT is visible in the upper left side of the screen during this time.

14. STBY BUTTON:  Stops the transponder from replying to any interrogations. Will

also power up the transponder unit.

1.PRALT SCREEN

PRESSURE ALTITUDE

2.FLT TIME SCREEN

FLIGHT TIME

3.ALTMON SCREEN

4.DENALT SCREEN

5.COUNT-UP SCREEN

6.COUNT-DOWN SCREEN

DENSITY ALTITUDE

COUNT-UP TIME

COUNT-DOWN TIME

GTX 330 FUNCTION SCREENS

FT FROM ASSIGNED

ABOVE/BELOW

1.  PRALT SCREEN:   Displays the PRESSURE ALTITUDE in feet / 10. Information

only, no user input. The ALT BUTTON must be clicked to display value. Serves as

the default screen.

2.  FLT TIME SCREEN:  Displays the flight time in hr:min:sec format. The timer is

automatically on when the aircraft is higher than 30 feet off the ground. It can be

stopped, or started, by the START/STOP BUTTON. Clicking the CLR BUTTON will

reset the value to 0.

3.  ALTMON SCREEN:  Displays the number of feet away from the set altitude. If the

aircraft is higher, the word ABOVE will be shown, if lower the word BELOW will be

shown. Can be turned off/on using the START/STOP BUTTON. ALT BUTTON must

be clicked to show altitude information.

4.  DENALT SCREEN:   Displays the OUTSIDE AMBIENT TEMPERATURE in

degrees Celsius and the DENSITY ALTITUDE in feet. The ALT BUTTON

must be clicked to show altitude information.

5.  COUNT-UP SCREEN:   Typical count-up timer displayed as hr:min:sec. Start and

stop using the START/STOP BUTTON. Reset using the CLR BUTTON.

6.  COUNT-DOWN TIMER:  Typical count-down timer displayed in an hr:min:sec

format. Start and stop using the START/STOP BUTTON. Reset using the CLR

BUTTON. Use number buttons at the bottom of the unit to enter data. The CURSOR

BUTTON must be clicked, and the timer reset before entering new count-down

numbers. The value cannot be changed when the count-down is active.

INSTRUCTIONS:

1.  Power on the unit using the ON, STBY, or ALT BUTTON. Turning the unit on with

the ALT BUTTON will allow you to receive altitude information without having to

click on the ALT BUTTON at a later time. Check to make sure the ON/OFF

ANNUNCIATOR is on.

(NOTE: The following steps prior to turning the transponder off are not

necessarily in a must-follow order.)

2.  Use the number 8 and 9 buttons to select the desired brightness of the screen.

3.  Click on the FUNC BUTTON to get to the FLIGHT TIMER screen. If desired, the

timer can be reset to 0 by clicking on the CLR BUTTON, or turned off by clicking the

START/STOP BUTTON. Default is on any time the aircraft is higher than 30 feet

from the ground.

4.  Click the FUNC BUTTON again to get to the ALTITUDE MONITOR screen. Click

on the START/STOP BUTTON to turn on/off the altitude monitoring function.

5.  Click the FUNC BUTTON again to get to the DENSITY ALTITUDE screen. There is

no user input per se, only information is displayed, and only displayed if the altitude

function is on (ALT BUTTON clicked).

6.  Click the FUNC BUTTON another time to get to the COUNT-UP TIMER screen.

Start/stop and/or reset the timer to 0 using the appropriate buttons.

7.  Click the FUNC BUTTON again to go to the COUNT-DOWN TIMER screen. Click

on the CURS BUTTON to allow for count-down data entry. A rectangle will appear

behind the first digit of the timer display. Click on any of the 9 numbers to enter that

number into the first digit location. This entry will cause the rectangle to be shifted to

the next digit to the right and the unit will be ready for the next number. This process

is repeated until all of the numbers have been entered. When the last number has been

entered, the rectangle will disappear and the number will be set as the active number.

Entering the last number will also set the TRANSPONDER CODE SET numbers back

to changing the code rather than the timer. Count-down data entry can be cancelled at

any time by clicking on the CURS BUTTON. You can also scroll in reverse order

through the timer numbers when entering data by clicking on the CLR BUTTON. The

unit will not allow you to enter minutes or seconds higher than 59.

8.  Set the transponder code using the TRANSPONDER CODE SET BUTTONS. Click

on any of the 7 number buttons to enter that number into the first digit. A rectangle

will appear behind the next digit indicating the unit is ready to accept the next number.

Repeat this process until the last number is entered, at which time the rectangle will

disappear and the 4 digit number will be set as the active code. Data entry can be

cancelled at any time by clicking on the CURS BUTTON. Clicking the CLR

BUTTON when adding code values will allow scrolling in reverse order through the

values. The code can be changed at any time by the above process except when

entering count-down data.

9.  Use the VFR BUTTON to change the transponder code directly to the VFR frequency

1200.

10. Click on the IDENT BUTTON if ATC asks for your ident.

11. Clicking the FUNC BUTTON at the count-down screen will "wrap around" to open

the default pressure altitude screen.

12. Turn the transponder unit off by clicking on the OFF BUTTON.